# Physical and Mathematical Theory of the Model — STCW Engine Room Simulator

Technical reference document. All formulas below are transcribed directly from engine_core.py, scenario_engine.py, exercise_eval.py, and coach_engine.py — they are the exact description of the running code, not retrospective approximations.

## 1. Model philosophy

The physics core (engine_core.py) is adapted and simplified from the NTNU academic project ship_in_transit_simulator (Børge Rokseth), from which the basic classes PidController and EulerInt were retained, along with the shaft equilibrium equation (shaft_eq). All ship hydrodynamics (surge/sway/yaw, environment, course) have been removed — for an ERS, only the propulsion and power generation system is of interest.

The default main engine is a Wärtsilä 6L26 (2160 kW); the diesel generators are Baudouin 6M26.3 (510 kW each) — actual specific consumption coefficients from the NTNU source.

All simulation uses explicit Euler integration with fixed step dt (default 0.1 s, 10 Hz):

x(t + dt) = x(t) + dx/dt · dt

EulerInt class does exactly this: integrate(x, dx) = x + dx·dt.

## 2. Dynamics of the main engine and propeller shaft

The model explicitly separates the engine's own speed (crankshaft, ω_motor) from the propeller shaft speed (ω_ax), coupled via a slip clutch. This allows the engine to accelerate/decelerate quickly at idle (low inertia) even when disengaged, while the propeller shaft has high inertia and responds slowly.

### 2.1 Speed regulator (governor)

Classic PID controller:

error = setpoint − measurement

error_i += error · dt  (bounded to [−1, 1])

d_error = (error − previous_error) / dt

output = Kp·error + Kd·d_error + Ki·error_i

For the main engine: Kp = 0.15, Ki = 0.02, Kd = 0. The target is target_omega = effective_throttle · ω_nominal, the measurement is ω_engine. The output (load_cmd, saturated to [−1, 1]) becomes the requested load percentage.

### 2.2 Main engine torque

torque_me = min( |load_perc| · P_available / (ω_motor + 0.1),

P_available / 5 · π/30 ) · sign(load_perc)

The term ω_motor + 0.1 avoids division by zero at zero speed; the second term limits maximum torque at start-up (approximation of a real diesel torque curve).

### 2.3 Engine shaft equation (crankshaft)

eq_me = (torque_me − friction_me · ω_motor) / gear_ratio_me

d(ω_motor)/dt = (eq_me − clutch_couple) / J_motor

J_motor (engine_inertia, default 2500) — the engine's own inertia, much smaller than the propeller's inertia, which is why the disengaged engine revs quickly.

### 2.4 Clutch (pneumatic coupling)

The clutch position does not instantly follow the command — it approaches progressively:

position(t+dt) = position(t) ± dt / ramp_time

with ramp_time = 4.0 s when engaging (ENGAGE_RAMP_S) and 1.5 s when disengaging (DISENGAGE_RAMP_S).

Torque transmitted through the clutch:

slip = ω_motor − ω_axle

clutch_torque = position · torque_capacity · sat( slip · 0.08, −1, 1 )

Engagement interlock: the engagement command is rejected if the engine is not running OR its speed is not in the idle band [0.12·ω_nominal, 0.35·ω_nominal] — exactly the real STCW check before engaging the propeller.

Excessive slip alarm (clutch_slip): triggered if the clutch is fully engaged (position ≥ 0.99) AND |slip| > 0.3·ω_nominal, sustained ≥ 15 consecutive seconds.

### 2.5 Propeller shaft equation

eq_hsg = (torque_hsg − friction_hsg · ω_ax) / gear_ratio_hsg

propeller_resistance = K_p · ω_ax · |ω_ax|  (quadratic, as in a real propeller)

d(ω_ax)/dt = (clutch_torque + eq_hsg − propeller_resistance) / J_propeller

J_propeller (propeller_inertia, default 6000) >> J_motor — hence the large inertia during ship acceleration/deceleration.

## 3. Operating modes and load distribution (PTO / MEC / PTI)

GEN:   P_disp_me = P_motor − hotel_load   P_disp_el = 0

MOTOR: P_disp_me = P_motor                P_disp_el = P_electrical − hotel_load

OFF:   P_disp_me = P_motor                P_disp_el = 0

Load distribution across sources depends on the mode — see MachineryMode.distribute_load() for the exact three branches.

## 4. Specific fuel consumption

Classical quadratic formula with real coefficients from the NTNU source:

specific_consumption(load_perc) = max(0, a·load_perc² + b·load_perc + c) / 3.6×10⁹  [kg/s per W]

load_perc is saturated at [0, 1.2] before calculation. Total fuel flow: fuel_rate = load_me · specific_consum(load_perc_me), accumulated over time for total session consumption.

## 5. Starting air system

- Tank with initial pressure 30 bar (AIR_MAX_BAR).

- Engine start is rejected if pressure is below 18 bar (AIR_MIN_START_BAR).

- A successful start consumes 6 bar (AIR_CONSUMED_PER_START_BAR).

- Refill: each powered air compressor adds 0.5 bar/s (COMPRESSOR_FILL_RATE_BAR_S); two compressors fill twice as fast.

## 6. Cooling and oil pressure

The cooling model is two-stage (seawater → heat exchanger → freshwater → engine):

cooling_frac = 1.0  if (SW active) AND (FW active)

= 0.5  if only one of the two is active

= 0.0  if neither

target_normal = 65 + 20 · min(1, ω_ax / ω_nominal)

target = target_normal + (160 − target_normal) · (1 − cooling_frac)

Actual temperature relaxes exponentially towards the target (first-order discrete form):

T(t+dt) = T(t) + (target − T(t)) · 0.05

This relaxation pattern (x += (target − x) · rate) is used throughout the model for any "slow" quantity (temperatures, tank levels).

Oil pressure with asymmetric rise vs. fall rates:

if pump running: P(t+dt) = P(t) + (4.2 − P(t)) · 0.10

otherwise:       P(t+dt) = P(t) + (0 − P(t)) · 0.20

## 7. Diesel generators and power bus

### 7.1 Local generator governor

start_ramp(t) = sat(t / 8.0, 0, 1)

droop = (droop_rpm_full_load · π/30) · load_perc  (if breaker closed)

target_ω = ω_nominal · ramp − droop

ω(t+dt) = ω(t) + (target_ω − ω(t)) · min(1, 6·dt)

droop_rpm_at_full_load = 20 rpm — typical droop governor speed drop at 100% load.

### 7.2 Synchronisation and paralleling

close_breaker() only succeeds if the generator is at rated speed within ±3% (at_rated_speed()). Otherwise, sync_fault is declared — the classic STCW "generator paralleling" training scenario.

### 7.3 Load sharing

share_i = demand · (P_nominal_i / ΣP_nominal_online)

If demand exceeds total online capacity → overload = True. If no generator is online and there is demand → blackout = True: bus loses all power, and main engine electric pumps fail regardless of their local switch.

## 8. Phase 1 — auxiliary systems and STCW faults

All quantities below use the same exponential relaxation pattern x += (target − x) · rate (section 6), or simple linear integration x += rate · dt for tank levels.

### 8.1 Thrust bearing

load_frac = min(1, ω_ax / ω_nominal)

target = 55 + 25 · load_frac

+ 25  if lubrication missing AND engine running

+ 35  if explicit fault injected (thrust_bearing_fault)

T(t+dt) = T(t) + (target − T(t)) · 0.03

Alarm: T > 85 °C.

### 8.2 Cylinder misfire and exhaust temperatures

baseline = 20 + 280 · min(1, |throttle|)  (if engine running, else 20)

target_i = baseline · 0.35  if i == faulty cylinder

= baseline · 1.08  if there IS a faulty cylinder but i ≠ it

= baseline          if no cylinder is faulty

exhaust_i(t+dt) = exhaust_i(t) + (target_i − exhaust_i(t)) · 0.08

Alarm: misfire_detected = engine running AND a misfire cylinder is set.

### 8.3 Turbocharger surging

Requires simultaneously: an injected fault (clogged air filter) AND sudden load variation at high speed:

d_throttle = |throttle(t) − throttle(t−dt)|

surge_now = turbo_fault AND engine running AND |throttle| > 0.55 AND (d_throttle/dt) > 0.05

if surge_now: surge_timer = 6.0

else if surge_timer > 0: surge_timer −= dt

Alarm: surge_timer > 0 (lasts 6 seconds from last trigger).

### 8.4 Fire in the separator compartment

Binary state with extinction conditional on both corrective actions:

if fire_active:

if QCV_closed AND CO2_released:

extinguishing_timer += dt

if extinguishing_timer >= 3.0: fire_active = False; separator → FAULT

else:

extinguishing_timer = 0  (any missing action → timer resets)

This strict AND (not OR) is intentionally pedagogical: a single measure (only fuel isolation OR only CO₂) is insufficient in a real compartment fire.

### 8.5 Sewage, bilge, fuel service tank, incinerator

All four use simple linear integration level += (fill − empty) · dt, saturated at [0, 100]%.

MARPOL violation (marpol_violation): triggered if sewage level remains above the sustained threshold for more than 30 seconds (_sewage_high_timer > 30) OR incinerator is over-temperature — a brief exceedance promptly corrected does not constitute a real discharge violation.

### 8.6 The forced alarm mechanism (forced_alarms)

All alarms calculated from real physics pass through a common filter (see §5 in the Developer Guide). A scenario can force an alarm (force_alarm(key)) for procedural training. While the key is in forced_alarms, the physical calculation in step() does not override it.

## 9. Phase 6 — Extended physics

Phase 6 is implemented in engine_core.py. These subsystems run every simulation tick alongside Phase 1 physics.

### 9.1 Dual HT/LT freshwater cooling circuits

Two separate freshwater circuits complement the base cooling model:

- fw_ht (high-temperature, jacket water cooling) — feeds engine cylinder cooling.

- fw_lt (low-temperature, central heat exchanger) — cools the HT return and lubricating oil.

lt_target = 38.0  if (sw_ok AND fw_lt_ok)

= 55.0  if only one active

= 85.0  if neither

The HT circuit raises cooling_water_temp further if fw_ht is inactive.

### 9.2 Fuel viscosity and preheater

if heater_ok (fuel_heater aux + bus powered + steam_pressure >= 2.0 bar):

fuel_viscosity = 12.0 + max(0, (130.0 − fuel_temp) · 0.35)

else:

fuel_viscosity = 3.5  (cold, low viscosity HFO approximation)

Alarm high_fuel_viscosity: engine running AND viscosity > 20.0 cSt.

### 9.3 Steam boiler

burner_steam_gen = 0.04  if (boiler_active AND bus_powered AND boiler_water_level > 15%)

= 0.0   otherwise

boiler_water_level += (water_refill − water_evaporation) · dt  [%, clipped 0–100]

Alarms: boiler_low_water_level (level < 20%), boiler_high_pressure (steam_pressure > 8.5 bar).

### 9.4 Turbogenerator

turbogenerator_running = (steam_pressure >= 5.0)

turbogenerator_power_kw = 120.0  if running  else 0.0

### 9.5 MARPOL Annex VI emissions

fuel_rate_kg_h = fuel_rate_kg_s · 3600

emissions_co2_kg_h = fuel_rate_kg_h · 3.114

sox_raw_kg_h = fuel_rate_kg_h · (2.0 · fuel_sulfur_pct / 100.0)

emissions_sox_kg_h = sox_raw_kg_h · (0.10  if scrubber_on  else 1.0)

emissions_nox_g_kwh = 14.5  if (running AND NOT scr_on)

= 2.8   if (running AND scr_on)

= 0.0   if not running

alarm emissions_violation: running AND (SOx > 2.5 kg/h OR NOx > 10.0 g/kWh)

## 10. Scenario Engine — fault injection signal shapes (Phase 4)

A continuous_trigger modifies a state variable in real time, according to 6 possible mathematical forms (t = time elapsed since trigger). The base_value is automatically captured from the current state on the first tick if not explicitly specified.

| Shape | Formula | Notes |
| --- | --- | --- |
| ramp | v(t) = start + (end−start) · t/duration | Remains at end value after expiry |
| step | v(t) = initial (t < delay), step_value (t ≥ delay) | Instantaneous switch |
| pulse | v(t) = pulse_val (t < width), base (t ≥ width) | Single pulse then returns to base |
| pulse_train | v(t) = pulse_val (t mod period < width), else base | Repetitive pulses |
| sine | v(t) = base + amplitude · sin(2π · freq · t) | Continuous oscillation |
| random | v(t) = base + noise (uniform); Random Walk: accumulates deviation | Noise simulation |

## 11. Evaluation engine — scoring formula

For each non-penalty criterion with score p, if satisfied, add p to total_awarded; the maximum possible is the sum of all positive scores, total_max.

final_score = clamp( round( 100 · total_awarded / total_max ), 0, 100 )

Penalty criteria subtract directly from total_awarded (do not contribute to total_max) — a single dangerous action can take the score below the raw positive total.

STCW segmented report: each criterion is labelled with an STCW category; for each category:

category_pct = clamp( 100 · (awarded_category − deductions_category) / max_category, 0, 100 )

"Completed" exercise: final_score ≥ 70 AND no penalty applied.

## 12. e-Coach — message trigger mechanism

Each rule (CoachRule) has a periodically evaluated Boolean condition, plus two temporal parameters:

- trigger_delay_s: the condition must remain true continuously for this duration before the message is emitted (avoids false alarms on fraction-of-a-second transients).

- cooldown_s: after issuing, the rule cannot re-trigger for this duration even if the condition remains true.

if condition(state) == True:

if condition_true_since == None: condition_true_since = now

if (now − condition_true_since) >= trigger_delay_s

AND (now − last_triggered_at) >= cooldown_s:

emit message; last_triggered_at = now

else:

condition_true_since = None

## 13. Known model limitations

This simulator is an educational platform, not a commercial class simulator (Kongsberg/Wärtsilä). Consciously assumed simplifications:

- Tank temperatures and levels use first-order exponential relaxation / linear integration — not full thermodynamic equations (no real heat capacity, no surface heat transfer model).

- There are no pressure wave patterns in fuel/air/oil circuits — pressures are scalar states with first-order dynamics.

- Propeller resistance is a simple quadratic model (K·ω·|ω|), without cavitation effects, non-uniform flow, or hull interaction.

- Turbogenerator power is computed and offsets the electrical bus demand in parallel with the diesel generators.

- Phase 1 models for tanks (sewage/bilge/fuel/incinerator) are linear rates/first-order relaxations, sufficient for STCW training but not full thermodynamics.

- e-Coach rules and exercise texts are in Romanian only, regardless of UI language.