# User Guide — Instructor

Your work interface: instructor.html. This guide describes exactly the controls in the application, functionally verified.

## 1. New session

The "Start new session" button sends the new_session action — clears the student's action log and resets the e-Coach, without restarting the physics engine. Use it between two different students on the same installation.

The "Reset completely" button sends the reset action — fully rebuilds the installation from scratch (engine, generators, electrical bus, all auxiliary units). Use it when you need a clean slate (e.g. after a catastrophic state during an exercise).

## 2. Uploading an STCW exercise

The exercise panel loads the list from scenarios/exercise_pack.json (35 exercises as of this release). By selecting an exercise, the app:

- Sends load_scenario with the complete associated JSON scenario.

- Resets the e-Coach and loads the rules specific to that scenario (if any).

- Starts the internal evaluation timer.

At the end of the exercise (or upon request), the evaluation (evaluate_exercise) runs — the result appears in a modal with score, STCW report by category, and timeline of the student's actions. The evaluation report labels respect the UI language (EN/RO).

## 3. Live fault injection (Phase 4)

This is the central panel for dynamic training: you can modify a plant state variable in real time using a signal shape of your choice. Each trigger is sent as an inject_event action with a continuous_trigger.

### 3.1 Available variables (functionally verified)

| Variable | What it represents |
| --- | --- |
| lube_oil_pressure | Lubrication oil pressure (bar) |
| cooling_water_temp | Cooling water temperature (°C) |
| air_pressure | Starting air pressure (bar) |
| thrust_bearing_temp | Thrust bearing temperature (°C) |
| sewage_tank_level | Sewage tank level (%) |
| bilge_level | Bilge level (%) |
| fuel_service_tank_level | Fuel service tank level (%) |
| incinerator_temp | Incinerator temperature (°C) |

### 3.2 Available signal shapes

| Shape | Parameters | Behaviour |
| --- | --- | --- |
| Ramp (ramp) | start value, end value, duration | Linear interpolation from start to end over the specified duration |
| Step (step) | initial value, step value, delay | Remains at initial value until delay, then switches instantly |
| Single pulse (pulse) | base value, pulse value, pulse width | Pulse value for the duration of the width, then returns to base — once |
| Pulse train (pulse_train) | base value, pulse value, period, pulse width | Repetitive pulses with the specified period |
| Sinusoidal (sine) | base value (opt.), amplitude, frequency (Hz) | Continuous sinusoidal oscillation around the base value |
| Random | base value, noise amplitude, "Random Walk" switch | Uniform noise around base; with Random Walk active, deviation accumulates progressively |

The general "duration" field sets the total trigger duration — upon expiration the variable returns to its base value (except for ramp, which remains at the final value).

### 3.3 Pedagogical good practices

- Use ramp for realistic progressive degradations (e.g. slow oil pressure drop) — useful for early failure recognition exercises.

- Use step for sudden, instantaneous failures (e.g. pump failure) — tests reaction time.

- Use sinusoidal/random to simulate instrument instability/noise without a real background fault — useful for testing whether the student can distinguish a normal fluctuation from a real fault.

- Combine with dedicated fault commands (section 4) for compound scenarios (e.g. ramp on thrust_bearing_temp + simultaneous lubrication fault).

## 4. Dedicated fault controls (beyond the trigger panel)

Some Phase 1 failures are injected via dedicated scenario actions — you can trigger them from the scenario panel or by editing a scenario file:

| Action | Effect |
| --- | --- |
| set_misfire (with cylinder: 0-5) | Triggers misfire on the indicated cylinder |
| set_thrust_bearing_fault | Activates thrust bearing failure |
| set_turbo_fault | Activates turbocharger surging predisposition (requires sudden load variation to manifest) |
| purifier_fire_start | Starts fire in the separator compartment |
| alarm_raised / alarm_cleared (with key) | Manually force/clear any alarm from the alarm panel |

## 5. Dashboard — monitoring

The instructor panel displays in real time: all active alarms, the status of each auxiliary unit, current e-Coach messages, and the student's action log. Use the action log to verify the actual order of the student's interventions, not just the final result.

## 6. STCW assessment and history

- STCW category report: each exercise criterion is labelled with an STCW competency; the report shows the percentage achieved in each category separately, not just the overall score. Labels are shown in the UI language (EN/RO).

- Score history: the results of previous exercises in the session remain available for comparison.

- Filterable log: the action timeline can be filtered for targeted review (e.g. only actions around a specific trigger).

- Session log export: the "Download CSV" button exports the full session event log (one row per second) for external analysis or replay.

## 7. Session and scenario reset — three distinct actions

| Action | Button in instructor.html | Effect |
| --- | --- | --- |
| new_session | "Start new session" | Clears action log + resets e-Coach. Physics engine and scenario continue. Use between students. |
| reset | "Reset completely" | Fully rebuilds engine, generators, bus and auxiliaries from scratch. Also clears log. |
| reset_scenario | "Reload current scenario" | Reloads the last loaded exercise/scenario (falls back to scenario_demo.json only if none was loaded yet this session). Accessible from the UI via the "Reload current scenario" button. |

## 8. P&ID page (pid-schema.html)

The P&ID page is accessible from the navigation bar on all pages. It shows the full plant piping diagram with live gauges.

## 9. Engine profile configuration (configurare.html)

The configuration page allows selecting the main engine and generator profiles for the session. It also supports importing new engine profiles via an Excel template (Template_Profiluri_Motoare_ERS01.xlsx). To add a custom engine profile:

- Download the Excel template from the configuration page.

- Fill in the engine parameters following the column headers.

- Upload the completed file — the server parses it and merges new profiles into engines_config.json.

## 10. Language

The interface defaults to English. The language selector in the navigation bar switches between English and Romanian. e-Coach messages and exercise titles/objectives/descriptions follow this setting as well.